## Frank: Merrill’s Atlas of Radiographic Positioning and Procedures, 11th Edition

## Chapter 01: Preliminary Steps in Radiography

## Test Bank

## MULTIPLE CHOICE

1.	The primary controlling factor of radiography contrast is:

| a. | mA |
| --- | --- |
| b. | time |
| c. | kVp |
| d. | SID |

ANS:	C	PTS:	1	DIF:	Level: Medium

OBJ:	Category: General		TOP:	Exam: 1

2.	The ability to visualize very small structures on a radiograph is termed:

| a. | contrast |
| --- | --- |
| b. | density |
| c. | distortion |
| d. | recorded detail |

ANS:	D	PTS:	1	DIF:	Level: Hard

OBJ:	Category: General		TOP:	Exam: 2

3.	Radiographs of the hands, wrist, feet, and toes are routinely displayed on the illuminator with the digits:

| a. | positioned toward the ceiling |
| --- | --- |
| b. | positioned toward the floor |
| c. | horizontal and pointed to the left |
| d. | horizontal and pointed to the right |

ANS:	A	PTS:	1	DIF:	Level: Medium

OBJ:	Category: General		TOP:	Exam: 1

4.	The total destruction of microorganisms is accomplished through the use of:

| a. | disinfectants |
| --- | --- |
| b. | sterilization |
| c. | germicides |
| d. | antiseptics |

ANS:	B	PTS:	1	DIF:	Level: Hard

OBJ:	Category: General		TOP:	Exam: 1

5.	Involuntary motion can be caused by which of the following?

1) peristalsis

2) spasm

3) breathing

| a. | 1 and 2 |
| --- | --- |
| b. | 1 and 3 |
| c. | 2 and 3 |
| d. | 1, 2, and 3 |

ANS:	A	PTS:	1	DIF:	Level: Medium

OBJ:	Category: Positioning		TOP:	Exam: None

6.	Voluntary motion resulting from lack of control can be caused by:

1) fear

2) age (child)

3) tremors

| a. | 1 and 2 |
| --- | --- |
| b. | 1 and 3 |
| c. | 2 and 3 |
| d. | 1, 2, and 3 |

ANS:	A	PTS:	1	DIF:	Level: Hard

OBJ:	Category: Positioning		TOP:	Exam: 2

7.	Radiographers can control voluntary motion by:

| a. | using a high kVp |
| --- | --- |
| b. | increasing the length of exposure time |
| c. | performing the examination in the recumbent position |
| d. | giving clear instruction to the patient |

ANS:	D	PTS:	1	DIF:	Level: Medium

OBJ:	Category: Positioning		TOP:	Exam: 1

8.	A decrease in technical factors may be required for a patient who has:

| a. | edema |
| --- | --- |
| b. | emphysema |
| c. | atelectasis |
| d. | advanced carcinoma |

ANS:	B	PTS:	1	DIF:	Level: Medium

OBJ:	Category: Positioning		TOP:	Exam: 1

9.	An increase in technical factors may be required to penetrate a part on a patient who has:

| a. | atrophy |
| --- | --- |
| b. | emphysema |
| c. | pleural effusion |
| d. | degenerative arthritis |

ANS:	C	PTS:	1	DIF:	Level: Hard

OBJ:	Category: Positioning		TOP:	Exam: 2

10.	All radiographs must be identified with which of the following?

1) radiographer’s name

2) patient’s name or ID number

3) right or left marker

| a. | 1 and 2 |
| --- | --- |
| b. | 1 and 3 |
| c. | 2 and 3 |
| d. | 1, 2, and 3 |

ANS:	C	PTS:	1	DIF:	Level: Medium

OBJ:	Category: Positioning		TOP:	Exam: 1

11.	The metric conversion of 40 inches is:

| a. | 16 cm |
| --- | --- |
| b. | 18 cm |
| c. | 90 cm |
| d. | 102 cm |

ANS:	D	PTS:	1	DIF:	Level: Medium

OBJ:	Category: General		TOP:	Exam: None

12.	The source of radiation in an x-ray tube (the anode) shall not be closer than:

| a. | 12 inches from the patient |
| --- | --- |
| b. | 15 inches from the patient |
| c. | 12 inches from the tabletop |
| d. | 15 inches from the tabletop |

ANS:	A	PTS:	1	DIF:	Level: Hard

OBJ:	Category: General		TOP:	Exam: 2

13.	Collimation of the x-ray beam prompts which of the following:

1) an increase in radiographic contrast

2) reduction of scatter and secondary radiation

3) reduction in radiation to the patient

| a. | 1 and 2 |
| --- | --- |
| b. | 1 and 3 |
| c. | 2 and 3 |
| d. | 1, 2, and 3 |

ANS:	D	PTS:	1	DIF:	Level: Medium

OBJ:	Category: Positioning		TOP:	Exam: 1

14.	Federal guidelines require gonad shielding for which of the following:

1) when the gonads lie in the primary beam

2) if the clinical objective of the exam is not compromised

3) when the patient has reasonable reproductive potential

| a. | 1 and 2 |
| --- | --- |
| b. | 1 and 3 |
| c. | 2 and 3 |
| d. | 1, 2, and 3 |

ANS:	D	PTS:	1	DIF:	Level: Medium

OBJ:	Category: Positioning		TOP:	Exam: 1

15.	Which of the following devices are considered IRs?

1) television monitor

2) Computed radiography image plate

3) cassette with film

| a. | 1 and 2 |
| --- | --- |
| b. | 1 and 3 |
| c. | 2 and 3 |
| d. | 1, 2, and 3 |

ANS:	C	PTS:	1	DIF:	Level: Medium

OBJ:	Category: General		TOP:	Exam: 3

16.	Recorded detail is primarily controlled by:

1) mA

2) screens

3) focal spot size

| a. | 1 and 2 |
| --- | --- |
| b. | 1 and 3 |
| c. | 2 and 3 |
| d. | 1, 2, and 3 |

ANS:	C	PTS:	1	DIF:	Level: Hard

OBJ:	Category: General		TOP:	Exam: 3

17.	If a bone is projected longer or shorter than it actually is on the radiographic image, it is known as:

| a. | angulation |
| --- | --- |
| b. | distortion |
| c. | geometry |
| d. | anatomic position |

ANS:	B	PTS:	1	DIF:	Level: Hard

OBJ:	Category: General		TOP:	Exam: 3

18.	Which of the following must be cleaned after every patient?

| a. | collimator |
| --- | --- |
| b. | tabletop |
| c. | gonad shields |
| d. | x-ray tube |

ANS:	B	PTS:	1	DIF:	Level: Medium

OBJ:	Category: General		TOP:	Exam: 3

19.	Which of the following is true when using computed radiography?

1) the IR could be opened for a few minutes without causing the stored image to be destroyed

2) collimation of the x-ray beam is not critical

3) final contrast and density adjustments of the radiographic image are done using a computer

| a. | 1 and 2 |
| --- | --- |
| b. | 1 and 3 |
| c. | 2 and 3 |
| d. | 1, 2, and 3 |

ANS:	B	PTS:	1	DIF:	Level: Hard

OBJ:	Category: General		TOP:	Exam: 3

20.	The phosphors in computed radiography IRs are very sensitive to:

| a. | kVp |
| --- | --- |
| b. | dust |
| c. | collimation |
| d. | scatter radiation |

ANS:	D	PTS:	1	DIF:	Level: Medium

OBJ:	Category: General		TOP:	Exam: 3

21.	Radiographers who are educated as "radiologist extenders" are known as:

1) physician assistants (PAs)

2) radiologist assistants (RAs)

3) radiology practitioner assistants (RPAs)

| a. | 1 and 2 |
| --- | --- |
| b. | 2 and 3 |
| c. | 2 and 3 |
| d. | 1, 2, and 3 |

ANS:	C	PTS:	1	DIF:	Level: Medium

OBJ:	Category: General		TOP:	Exam: 2

22.	Blood and body fluid recommendations are issued by the:

| a. | CDC |
| --- | --- |
| b. | ASRT |
| c. | ARRT |
| d. | JCAHO |

ANS:	A	PTS:	1	DIF:	Level: Medium

OBJ:	Category: General		TOP:	Exam: 1

23.	Each radiology department must write age-specific competencies to meet the standards of the:

| a. | CDC |
| --- | --- |
| b. | ASRT |
| c. | ARRT |
| d. | JCAHO |

ANS:	D	PTS:	1	DIF:	Level: Hard

OBJ:	Category: General		TOP:	Exam: 2

24.	Gonad shielding is required when the gonads lie within ____ cm of the primary x-ray field.

| a. | 3 |
| --- | --- |
| b. | 5 |
| c. | 6 |
| d. | 8 |

ANS:	B	PTS:	1	DIF:	Level: Medium

OBJ:	Category: General		TOP:	Exam: 1

25.	Which of the following radiographic examinations would give a male patient the highest gonad dose?

| a. | limb |
| --- | --- |
| b. | skull |
| c. | pelvis |
| d. | lumbar spine |

ANS:	C	PTS:	1	DIF:	Level: Hard

OBJ:	Category: General		TOP:	Exam: 1

26.	Which of the following radiographic examinations would give a female patient the highest gonad dose?

| a. | limb |
| --- | --- |
| b. | skull |
| c. | pelvis |
| d. | lumbar spine |

ANS:	D	PTS:	1	DIF:	Level: Hard

OBJ:	Category: General		TOP:	Exam: 1