Chapter 2

## Simple Comparative Experiments

# Solutions

2.1.	The Minitab output for a random sample of data is shown below.  Some of the quantities are missing.  Compute the values of the missing quantities.

| Variable | N | Mean | SE Mean | Std. Dev. | Sum |
| --- | --- | --- | --- | --- | --- |
| Y | 15 | ? | 0.159 | ? | 399.851 |

Mean = 26.657	Std. Dev. = 0.616

2.2.	The Minitab output for a random sample of data is shown below.  Some of the quantities are missing.  Compute the values of the missing quantities.

| Variable | N | Mean | SE Mean | Std. Dev. | Variance | Minimum | Maximum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Y | 11 | 19.96 | ? | 3.12 | ? | 15.94 | 27.16 |

SE Mean = 0.941	Variance = 9.73

2.3.	Consider the Minitab output shown below.

| One-Sample Z |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test of mu = 30 vs not = 30 |  |  |  |  |  |
| The assumed standard deviation = 1.2 |  |  |  |  |  |
| N | Mean | SE Mean | 95% CI | Z | P |
| 16 | 31.2000 | 0.3000 | (30.6120, 31.7880) | ? | ? |

(a)	Fill in the missing values in the output.  What conclusion would you draw?

Z = 4	P = 0.00006

(b)	Is this a one-sided or two-sided test?

Two-sided.

(c)	Use the output and the normal table to find a 99 percent CI on the mean.

CI = 30.42725, 31.97275

(d)	What is the P-value if the alternative hypothesis is H1: µ > 30?

P-value = 0.00003

2.4.	Suppose that we are testing H0: µ = µ0 versus H1: µ > µ0.  Calcualte the P-value for the following observed values of the test statistic:

(a)	Z0 = 2.45 	P-value = 0.00714

(b)	Z0 = -1.53 	P-value = 0.93699

(c)	Z0 = 2.15 	P-value = 0.01578

(d)	Z0 = 1.95 	P-value = 0.02559

(e)	Z0 = -0.25 	P-value = 0.59871

2.5.	Suppose that we are testing H0: µ = µ0 versus H1: µ ≠ µ0.  Calcualte the P-value for the following observed values of the test statistic:

(a)	Z0 = 2.25	P-value = 0.02445

(b)	Z0 = 1.55 	P-value = 0.12114

(c)	Z0 = 2.10 	P-value = 0.03573

(d)	Z0 = 1.95 	P-value = 0.05118

(e)	Z0 = -0.10 	P-value = 0.92034

2.6.	Suppose that we are testing H0: µ = µ0 versus H1: µ ≠ µ0 with a sample size of n = 9.  Calculate bounds on the P-value for the following observed values of the test statistic:

(a)	t0 = 2.48	Table P-value = 0.02, 0.05	Cumputer P-value = 0.03811

(b)	t0 = -3.95 	Table P-value = 0.002, 0.005	Cumputer P-value = 0.00424

(c)	t0 = 2.69 	Table P-value = 0.02, 0.05	Cumputer P-value = 0.02750

(d)	t0 = 1.88 	Table P-value = 0.05, 0.10	Cumputer P-value = 0.09691

(e)	t0 = -1.25 	Table P-value = 0.20, 0.50	Cumputer P-value = 0.24663

2.7.	Suppose that we are testing H0: µ = µ0 versus H1: µ > µ0 with a sample size of n = 13.  Calculate bounds on the P-value for the following observed values of the test statistic:

(a)	t0 = 2.35	Table P-value = 0.01, 0.025	Cumputer P-value = 0.01836

(b)	t0 = 3.55 	Table P-value = 0.001, 0.0025	Cumputer P-value = 0.00200

(c)	t0 = 2.00 	Table P-value = 0.025, 0.005	Cumputer P-value = 0.03433

(d)	t0 = 1.55 	Table P-value = 0.05, 0.10	Cumputer P-value = 0.07355

2.8.	Consider the Minitab output shown below.

| One-Sample T: Y |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test of mu = 25 vs > 25 |  |  |  |  |  |  |  |
| Variable | N | Mean | Std. Dev. | SE Mean | 95% Lower Bound | T | P |
| Y | 12 | 25.6818 | ? | 0.3360 | ? | ? | 0.034 |

(a)	How many degrees of freedom are there on the t-test statistic?

11

(b)	Fill in the missing information.

Std. Dev. = 1.1639	95% Lower Bound = 2.0292

2.9.	Consider the Minitab output shown below.

| Two-Sample T-Test and CI: Y1, Y2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Two-sample T for Y1 vs Y2 |  |  |  |  |
|  | N | Mean | Std. Dev. | SE Mean |
| Y1 | 20 | 50.19 | 1.71 | 0.38 |
| Y2 | 20 | 52.52 | 2.48 | 0.55 |
| Difference = mu (X1) – mu (X2) |  |  |  |  |
| Estimate for difference: -2.33341 |  |  |  |  |
| 95% CI for difference: (-3.69547, -0.97135) |  |  |  |  |
| T-Test of diffence = 0 (vs not = ) : T-Value = -3.47 |  |  |  |  |
| P-Value = 0.01 DF = 38 |  |  |  |  |
| Both use Pooled Std. Dev. = 2.1277 |  |  |  |  |

(a)	Can the null hypothesis be rejected at the 0.05 level?  Why?

Yes, the P-Value of 0.001 is much less than 0.05.

(b)	Is this a one-sided or two-sided test?

Two-sided.

(c)	If the hypothesis had been H0: µ1 - µ2 = 2 versus H1: µ1 - µ2 ≠ 2 would you reject the null hypothesis at the 0.05 level?

Yes.

(d)	If the hypothesis had been H0: µ1 - µ2 = 2 versus H1: µ1 - µ2 < 2 would you reject the null hypothesis at the 0.05 level?  Can you answer this question without doing any additional calculations?  Why?

Yes.  No additional calculations are required because the test becomes more significant with the change from -2.33341 to -4.33341.

(e)	Use the output and the t table to find a 95 percent upper confidence bound on the difference in means?

95% upper confidence bound = -1.21.

(f)	What is the P-value if the alternatie hypotheses are H0: µ1 - µ2 = 2 versus H1: µ1 - µ2 ≠ 2?

P-value = 1.4E-07.

2.10.	Consider the Minitab output shown below.

| One-Sample T: Y |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test of mu = 91 vs. not = 91 |  |  |  |  |  |  |  |
| Variable | N | Mean | Std. Dev. | SE Mean | 95% CI | T | P |
| Y | 25 | 92.5805 | ? | 0.4675 | (91.6160, ? ) | 3.38 | 0.002 |

(a)	Fill in the missing values in the output.  Can the null hypothesis be rejected at the 0.05 level?  Why?

Std. Dev. = 2.3365	UCI = 93.5450

Yes, the null hypothesis can be rejected at the 0.05 level because the P-value is much lower at 0.002.

(b)	Is this a one-sided or two-sided test?

Two-sided.

(c)	If the hypothesis had been H0: µ = 90 versus H1: µ ≠ 90 would you reject the null hypothesis at the 0.05 level?

Yes.

(d)	Use the output and the t table to find a 99 percent two-sided CI on the mean.

CI = 91.2735, 93.8875

(e)	What is the P-value if the alternative hypothesis is H1: µ > 91?

P-value = 0.001.

2.11.	The diameters of steel shafts produced by a certain manufacturing process should have a mean diameter of 0.255 inches.  The diameter is known to have a standard deviation of s = 0.0001 inch.  A random sample of 12 shafts has an average diameter of 0.2545 inches.

(a)	Set up the appropriate hypotheses on the mean m.

H0:  m = 0.255		H1:  m ¹ 0.255

(b)	Test these hypotheses using a = 0.05.  What are your conclusions?

n = 12,   s = 0.0001,  = 0.2545

z0=y−m0sn=0.2545−0.2550.000112=-17.32

Since z0.025 = 1.96, reject H0.

(c)	Find the P-value for this test.  P = 3.504x10-67

(d)	Construct a 95 percent confidence interval on the mean shaft diameter.

The 95% confidence interval is

y−zα2σn≤μ≤y+zα2σn

0.2545−(1.96)0.000112≤m≤0.2545+(1.96)0.000112

0.254443≤m≤0.254556

2.12.	The breaking strength of a fiber is required to be at least 150 psi. Past experience has indicated that the standard deviation of breaking strength is s = 3 psi. A random sample of four specimens is tested.  The results are y1=145, y2=153, y3=150 and y4=147.

(a)	State the hypotheses that you think should be tested in this experiment.

H0:  m = 150          H1:  m > 150

(b)	Test these hypotheses using a = 0.05.  What are your conclusions?

n = 4,   s = 3, = 1/4  (145 + 153 + 150 + 147) = 148.75

Since z0.05 = 1.645, do not reject.

(c)	Find the P-value for the test in part (b).

From the z-table:  P≃1−0.7967+2/30.7995−0.7967=0.2014

(d)	Construct a 95 percent confidence interval on the mean breaking strength.

The 95% confidence interval is

y−zα2σn≤μ≤y+zα2σn148.75−1.963/2≤μ≤148.75+1.963/2

2.13.	The viscosity of a liquid detergent is supposed to average 800 centistokes at 25°C.  A random sample of 15 batches of detergent is collected, and the average viscosity is 812.  Suppose we know that the standard deviation of viscosity is s = 25 centistokes.

(a)	State the hypotheses that should be tested.

H0:  m = 800 		H1:  m ¹ 800

(b)	Test these hypotheses using a = 0.05.  What are your conclusions?

z0=y−m0sn=812−8002515=1.859 	Since z/2 = z0.025 = 1.96, do not reject.

(c)	What is the P-value for the test?        P=2(0.03144)=0.06288

(d)	Find a 95 percent confidence interval on the mean.

The 95% confidence interval is

y−zα2σn≤μ≤y+zα2σn

812−1.9625/15≤m≤812+1.9625/15812−12.6517≤m≤812+12.6517799.3483≤m≤824.6517

2.14.	A normally distributed random variable has an unknown mean m and a known variance s2 = 9.  Find the sample size required to construct a 95 percent confidence interval on the mean, that has total length of 1.0.

Since y ~ N(m,9), a 95% two-sided confidence interval on m is

y−zα2sn≤m≤y+zα2sn

y−(1.96)3n≤m≤y+(1.96)3n

If the total interval is to have width 1.0, then the half-interval has width 0.5.  Since z/2 = z0.025 = 1.96,

1.963/n=0.5n=1.963/0.5=11.76n=11.762=138.30≃139

2.15.	The time to repair an electronic instrument is a normally distributed random variable measured in hours.  The repair time for 16 such instruments chosen at random are as follows:

| Hours |  |  |  |
| --- | --- | --- | --- |
| 159 | 280 | 101 | 212 |
| 224 | 379 | 179 | 264 |
| 222 | 362 | 168 | 250 |
| 149 | 260 | 485 | 170 |

(a)	You wish to know if the mean repair time exceeds 205 hours.  Set up appropriate hypotheses for investigating this issue.

H0:  m = 205		H1:  m > 205

(b)	Test the hypotheses you formulated in part (a). What are your conclusions?  Use a = 0.05.

= 241.50

S2 =146202 / (16 - 1) = 9746.80

t0=y−m0Sn=241.50−20598.7316=1.479

since t0.05,15 = 1.753; do not reject H0

Minitab Output

T-Test of the Mean

Test of mu = 205.0 vs mu > 205.0

Variable     N      Mean    StDev   SE Mean        T          P

Hours       16     241.5     98.7      24.7     1.48       0.080

T Confidence Intervals

Variable     N      Mean    StDev  SE Mean       95.0 % CI

Hours       16     241.5     98.7     24.7  (   188.9,   294.1)

(c)	Find the P-value for this test.  P=0.080

(d)	Construct a 95 percent confidence interval on mean repair time.

The 95% confidence interval is

188.9≤μ≤294.1

2.16.	The shelf life of a carbonated beverage is of interest.  Ten bottles are randomly selected and tested, and the following results are obtained:

| Days |  |
| --- | --- |
| 108 | 138 |
| 124 | 163 |
| 124 | 159 |
| 106 | 134 |
| 115 | 139 |

(a)	We would like to demonstrate that the mean shelf life exceeds 120 days.  Set up appropriate hypotheses for investigating this claim.

H0:  m = 120		H1:  m > 120

(b)	Test these hypotheses using a = 0.01.  What are your conclusions?

= 131

S2 = 3438 / 9 = 382

t0=y−μ0S/n=131−12019.54/10=1.78

since t0.01,9 = 2.821; do not reject H0

Minitab Output

T-Test of the Mean

Test of mu = 120.00 vs mu > 120.00

Variable     N      Mean    StDev   SE Mean        T          P

Shelf Life  10    131.00    19.54      6.18     1.78      0.054

T Confidence Intervals

Variable     N      Mean    StDev  SE Mean       99.0 % CI

Shelf Life  10    131.00    19.54     6.18  (  110.91,  151.09)

(c)	Find the P-value for the test in part (b).  P=0.054

(d)	Construct a 99 percent confidence interval on the mean shelf life.

The 99% confidence interval is  with a = 0.01.

2.17.	Reconsider the repair time data in Problem 2.15.  Can repair time, in your opinion, be adequately modeled by a normal distribution?

The normal probability plot below does not reveal any serious problem with the normality assumption.

2.18.	Consider the shelf life data in Problem 2.16.  Can shelf life be described or modeled adequately by a normal distribution?  What effect would violation of this assumption have on the test procedure you used in solving Problem 2.16?

A normal probability plot, obtained from Minitab, is shown.  There is no reason to doubt the adequacy of the normality assumption.  If shelf life is not normally distributed, then the impact of this on the t-test in problem 2.16 is not too serious unless the departure from normality is severe.

2.19.	Two types of plastic are suitable for use by an electronic calculator manufacturer.  The breaking strength of this plastic is important.  It is known that s1 = s2 = 1.0 psi.  From random samples of n1 = 10 and n2 = 12 we obtain 1 = 164.5 and 2 = 155.0.  The company will not adopt plastic 1 unless its breaking strength exceeds that of plastic 2 by at least 10 psi.  Based on the sample information, should they use plastic 1?  In answering this question, set up and test appropriate hypotheses using a = 0.01.  Construct a 99 percent confidence interval on the true mean difference in breaking strength.

H0:  m1 - m2 =10   	H1:   m1 - m2 >10

y1=164.5s1=1n1=10	y2=155.0σ2=1n2=10

z0=y1−y2−10s12n1+s22n2=164.5−155.0−101210+1212=-1.17

z0.01 = 2.325; do not reject

The  99 percent confidence interval is

y1−y2−zα2σ12n1+σ22n2≤μ1−μ2≤y1−y2+zα2σ12n1+σ22n2

(164.5−155.0)−(2.575)1210+1212≤m1−m2≤(164.5−155.0)+(2.575)1210+1212

8.40≤m1−m2≤10.60

2.20.	Two machines are used for filling plastic bottles with a net volume of 16.0 ounces.  The filling processes can be assumed to be normal, with standard deviation of s1 = 0.015 and s2 = 0.018.  The quality engineering department suspects that both machines fill to the same net volume, whether or not this volume is 16.0 ounces.  An experiment is performed by taking a random sample from the output of each machine.

| Machine 1 |  | Machine 2 |  |
| --- | --- | --- | --- |
| 16.03 | 16.01 | 16.02 | 16.03 |
| 16.04 | 15.96 | 15.97 | 16.04 |
| 16.05 | 15.98 | 15.96 | 16.02 |
| 16.05 | 16.02 | 16.01 | 16.01 |
| 16.02 | 15.99 | 15.99 | 16.00 |

(a)	State the hypotheses that should be tested in this experiment.

H0:  m1 = m2		H1:  m1 ¹ m2

(b)	Test these hypotheses using a=0.05.  What are your conclusions?

z0.025 = 1.96; do not reject

(c)	What is the P-value for the test?  P = 0.1770

(d)	Find a 95 percent confidence interval on the difference in the mean fill volume for the two machines.

The 95% confidence interval is

y1−y2−zα2σ12n1+σ22n2≤μ1−μ2≤y1−y2+zα2σ12n1+σ22n2

−0.0045≤μ1−μ2≤0.0245

2.21.	A new filtering device is installed in a chemical unit.  Before its installation, a random sample yielded the following information about the percentage of impurity:  1 = 12.5,  =101.17, and n1 = 10.  After installation, a random sample yielded 2 = 10.2,  = 94.73, n2 = 9.

(a)	Can you concluded that the two variances are equal?  Use a = 0.05.

H0:s12=s22H1:s12¹s22F0.025,9,8=4.36F0=S12S22=101.1794.73=1.07

Do Not Reject.  Assume that the variances are equal.

(b)	Has the filtering device reduced the percentage of impurity significantly?  Use a = 0.05.

H0:m1=m2H1:m1>m2Sp2=n1−1S12+n2−1S22n1+n2−2=10−1101.17+9−194.7310+9−2=98.1394Sp=9.9065t0=y1−y2Sp1n1+1n2=12.5−10.29.9065110+19=0.5053t0.05,17=1.740

Do not reject.  There is no evidence to indicate that the new filtering device has affected the mean.

2.22.	The following are the burning times (in minutes) of chemical flares of two different formulations. The design engineers are interested in both the means and variance of the burning times.

| Type 1 |  | Type 2 |  |
| --- | --- | --- | --- |
| 65 | 82 | 64 | 56 |
| 81 | 67 | 71 | 69 |
| 57 | 59 | 83 | 74 |
| 66 | 75 | 59 | 82 |
| 82 | 70 | 65 | 79 |

- Test the hypotheses that the two variances are equal.  Use a = 0.05.

S1=9.264S2=9.367

F0=S12S22=85.8287.73=0.98

F0.025,9,9=4.03     F0.975,9,9=1F0.025,9,9=14.03=0.248  Do not reject.

- Using the results of (a), test the hypotheses that the mean burning times are equal.  Use a = 0.05.  What is the P-value for this test?

Sp2=(n1−1)S12+(n2−1)S22n1+n2−2=1561.9518=86.775Sp=9.32t0=y1−y2Sp1n1+1n2=70.4−70.29.32110+110=0.048

t0.025,18=2.101   Do not reject.

From the computer output, t=0.05; do not reject.  Also from the computer output P=0.96

Minitab Output

Two Sample T-Test and Confidence Interval

Two sample T for Type 1 vs Type 2

N      Mean     StDev   SE Mean

Type 1  10     70.40      9.26       2.9

Type 2  10     70.20      9.37       3.0

95% CI for mu Type 1 - mu Type 2: ( -8.6,  9.0)

T-Test mu Type 1 = mu Type 2 (vs not =): T = 0.05  P = 0.96  DF = 18

Both use Pooled StDev = 9.32

- Discuss the role of the normality assumption in this problem.  Check the assumption of normality for both types of flares.

The assumption of normality is required in the theoretical development of the t-test.  However, moderate departure from normality has little impact on the performance of the t-test.  The normality assumption is more important for the test on the equality of the two variances.  An indication of nonnormality would be of concern here.  The normal probability plots shown below indicate that burning time for both formulations follow the normal distribution.

2.23.	An article in Solid State Technology, "Orthogonal Design of Process Optimization and Its Application to Plasma Etching" by G.Z. Yin and D.W. Jillie (May, 1987) describes an experiment to determine the effect of C2F6 flow rate on the uniformity of the etch on a silicon wafer used in integrated circuit manufacturing.  Data for two flow rates are as follows:

| C2F6 | Uniformity Observation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (SCCM) | 1 | 2 | 3 | 4 | 5 | 6 |
| 125 | 2.7 | 4.6 | 2.6 | 3.0 | 3.2 | 3.8 |
| 200 | 4.6 | 3.4 | 2.9 | 3.5 | 4.1 | 5.1 |

(a)	Does the C2F6 flow rate affect average etch uniformity?  Use a = 0.05.

No, C2F6 flow rate does not affect average etch uniformity.

Minitab Output

Two Sample T-Test and Confidence Interval

Two sample T for Uniformity

Flow Rat    N      Mean     StDev   SE Mean

125         6     3.317     0.760      0.31

200         6     3.933     0.821      0.34

95% CI for mu (125) - mu (200): ( -1.63,  0.40)

T-Test mu (125) = mu (200) (vs not =): T = -1.35  P = 0.21  DF = 10

Both use Pooled StDev = 0.791

(b)	What is the P-value for the test in part (a)?  From the Minitab output, P=0.21

(c)	Does the C2F6 flow rate affect the wafer-to-wafer variability in etch uniformity?  Use a = 0.05.

Do not reject; C2F6 flow rate does not affect wafer-to-wafer variability.

- Draw box plots to assist in the interpretation of the data from this experiment.

The box plots shown below indicate that there is little difference in uniformity at the two gas flow rates.  Any observed difference is not statistically significant.  See the t-test in part (a).

2.24.	Front housings for cell phones are manufactured in an injection molding process.  The time the part is allowed to cool in the mold before removal is thought to influence the occurrence of a particularly troublesome cosmetic defect, flow lines, in the finished housing.  After manufacturing, the housings are inspected visually and assigned a score between 1 and 10 based on their appearance, with 10 corresponding to a perfect part and 1 corresponding to a completely defective part.  An experiment was conducted using two cool-down times, 10 seconds and 20 seconds, and 20 housings were evaluated at each level of cool-down time.  The data are shown below.

| 10 Seconds |  | 20 Seconds |  |
| --- | --- | --- | --- |
| 1 | 3 | 7 | 6 |
| 2 | 6 | 8 | 9 |
| 1 | 5 | 5 | 5 |
| 3 | 3 | 9 | 7 |
| 5 | 2 | 5 | 4 |
| 1 | 1 | 8 | 6 |
| 5 | 6 | 6 | 8 |
| 2 | 8 | 4 | 5 |
| 3 | 2 | 6 | 8 |
| 5 | 3 | 7 | 7 |

(a)	Is there evidence to support the claim that the longer cool-down time results in fewer appearance defects?  Use a = 0.05.

From the analysis shown below, there is evidence that the longer cooll-down time results in fewer appearance defects.

Minitab Output

Two-Sample T-Test and CI: 10 seconds, 20 seconds

Two-sample T for 10 seconds vs 20 seconds

N      Mean     StDev   SE Mean

10 secon  20      3.35      2.01      0.45

20 secon  20      6.50      1.54      0.34

Difference = mu 10 seconds - mu 20 seconds

Estimate for difference:  -3.150

95% upper bound for difference: -2.196

T-Test of difference = 0 (vs <): T-Value = -5.57  P-Value = 0.000  DF = 38

Both use Pooled StDev = 1.79

(b)	What is the P-value for the test conducted in part (a)? From the Minitab output,  P = 0.000

(c)	Find a 95% confidence interval on the difference in means.  Provide a practical interpretation of this interval.

From the Minitab output, .  This lower confidence bound is less than 0.  The two samples are different.  The 20 second cooling time gives a cosmetically better housing.

(d)	Draw dot diagrams to assist in interpreting the results from this experiment.

(e)	Check the assumption of normality for the data from this experiment.

There are no significant departures from normality.

2.25.	Photoresist is a light-sensitive material applied to semiconductor wafers so that the circuit pattern can be imaged on to the wafer.  After application, the coated wafers are baked to remove the solvent in the photoresist mixture and to harden the resist. Here are measurements of photoresist thickness (in kÅ) for eight wafers baked at two different temperatures. Assume that all of the runs were made in random order.

| 95 ºC | 100 ºC |  |
| --- | --- | --- |
| 11.176 |  | 5.263 |
| 7.089 |  | 6.748 |
| 8.097 |  | 7.461 |
| 11.739 |  | 7.015 |
| 11.291 |  | 8.133 |
| 10.759 |  | 7.418 |
| 6.467 |  | 3.772 |
| 8.515 |  | 8.963 |

(a)	Is there evidence to support the claim that the higher baking temperature results in wafers with a lower mean photoresist thickness?  Use a = 0.05.

H0:m1=m2H1:m1>m2Sp2=n1−1S12+n2−1S22n1+n2−2=8−12.08642+8−11.640428+8−2=3.5220Sp=1.8767t0=y1−y2Sp1n1+1n2=9.3916−6.84661.876718+18=2.7122t0.05,14=1.761

Since t0.05,14 = 1.761, reject H0.  There appears to be a lower mean thickness at the higher temperature.  This is also seen in the computer output.

Minitab Output

Two-Sample T-Test and CI: Thickness, Temp

Two-sample T for Thick@95 vs Thick@100

N      Mean     StDev   SE Mean

Thick@95   8      9.39      2.09      0.74

Thick@100  8      6.85      1.64      0.58

Difference = mu Thick@95 - mu Thick@100

Estimate for difference:  2.545

95% lower bound for difference: 0.892

T-Test of difference = 0 (vs >): T-Value = 2.71  P-Value = 0.008  DF = 14

Both use Pooled StDev = 1.8767

(b)	What is the P-value for the test conducted in part (a)?  P = 0.008

(c)	Find a 95% confidence interval on the difference in means.  Provide a practical interpretation of this interval.

From the computer output the 95% lower confidence bound is 1 - 2  0.892.  This lower confidence bound is greater than 0; therefore, there is a difference in the two temperatures on the thickness of the photoresist.

(d)	Draw dot diagrams to assist in interpreting the results from this experiment.

(e)	Check the assumption of normality of the photoresist thickness.

There are no significant deviations from the normality assumptions.

(f)	Find the power of this test for detecting an actual difference in means of 2.5 kÅ.

Minitab Output

Power and Sample Size

2-Sample t Test

Testing mean 1 = mean 2 (versus not =)

Calculating power for mean 1 = mean 2 + difference

Alpha = 0.05  Sigma = 1.8767

Sample

Difference    Size   Power

2.5       8  0.697946

(g)	What sample size would be necessary to detect an actual difference in means of 1.5 kÅ with a power of at least 0.9?.

Minitab Output

Power and Sample Size

2-Sample t Test

Testing mean 1 = mean 2 (versus not =)

Calculating power for mean 1 = mean 2 + difference

Alpha = 0.05  Sigma = 1.8767

Sample  Target  Actual

Difference    Size   Power   Power

1.5      34  0.9000  0.900990

This result makes intuitive sense.  More samples are needed to detect a smaller difference.

2.26.	Twenty observations on etch uniformity on silicon wafers are taken during a qualification experiment for a plasma etcher.  The data are as follows:

| Etch Uniformity |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5.34 | 6.65 | 4.76 | 5.98 | 7.25 |
| 6.00 | 7.55 | 5.54 | 5.62 | 6.21 |
| 5.97 | 7.35 | 5.44 | 4.39 | 4.98 |
| 5.25 | 6.35 | 4.61 | 6.00 | 5.32 |

(a)	Construct a 95 percent confidence interval estimate of 2.

n−1S2cα/2,n−12≤s2≤n−1S2c1−α/2,n−1220−10.87941232.852≤s2≤20−10.8794128.9070.4473≤s2≤1.6497

- Test the hypothesis that s2 = 1.0.  Use  = 0.05.  What are your conclusions?

H0:s2=1H1:s2¹1

c02=n−1S2s02=14.6939

c0.025,192=32.852    c0.975,192=8.907

Do not reject.  There is no evidence to indicate that .

- Discuss the normality assumption and its role in this problem.

The normality assumption is much more important when analyzing variances then when analyzing means.  A moderate departure from normality could cause problems with both statistical tests and confidence intervals.  Specifically, it will cause the reported significance levels to be incorrect.

- Check normality by constructing a normal probability plot.  What are your conclusions?

The normal probability plot indicates that there is not a serious problem with the normality assumption.

2.27.	An article in the Journal of Strain Analysis (vol.18, no. 2, 1983) compares several procedures for predicting the shear strength for steel plate girders.  Data for nine girders in the form of the ratio of predicted to observed load for two of these procedures, the Karlsruhe and Lehigh methods, are as follows:

| Girder | Karlsruhe Method | Lehigh Method | Difference | Difference^2 |
| --- | --- | --- | --- | --- |
| S1/1 | 1.186 | 1.061 | 0.125 | 0.015625 |
| S2/1 | 1.151 | 0.992 | 0.159 | 0.025281 |
| S3/1 | 1.322 | 1.063 | 0.259 | 0.067081 |
| S4/1 | 1.339 | 1.062 | 0.277 | 0.076729 |
| S5/1 | 1.200 | 1.065 | 0.135 | 0.018225 |
| S2/1 | 1.402 | 1.178 | 0.224 | 0.050176 |
| S2/2 | 1.365 | 1.037 | 0.328 | 0.107584 |
| S2/3 | 1.537 | 1.086 | 0.451 | 0.203401 |
| S2/4 | 1.559 | 1.052 | 0.507 | 0.257049 |
|  |  | Sum = | 2.465 | 0.821151 |
|  |  | Average = | 0.274 |  |

- Is there any evidence to support a claim that there is a difference in mean performance between the two methods?  Use  = 0.05.

H0:m1=m2H1:m1¹m2  or equivalently  H0:μd=0H1:μd≠0

, reject the null hypothesis.

Minitab Output

Paired T-Test and Confidence Interval

Paired T for Karlsruhe - Lehigh

N      Mean     StDev   SE Mean

Karlsruh          9    1.3401    0.1460    0.0487

Lehigh             9    1.0662    0.0494    0.0165

Difference       9    0.2739    0.1351    0.0450

95% CI for mean difference: (0.1700, 0.3777)

T-Test of mean difference = 0 (vs not = 0): T-Value = 6.08  P-Value = 0.000

- What is the P-value for the test in part (a)?

P=0.0002

- Construct a 95 percent confidence interval for the difference in mean predicted to observed load.

d−tα2,n−1Sdn≤μd≤d+tα2,n−1Sdn0.274−2.3060.1359≤μd≤0.274+2.3060.13590.17023≤μd≤0.37777

- Investigate the normality assumption for both samples.

The normal probability plots of the observations for each method follows.  There are no serious concerns with the normality assumption, but there is an indication of a possible outlier (1.178) in the Lehigh method data.

- Investigate the normality assumption for the difference in ratios for the two methods.

- Discuss the role of the normality assumption in the paired t-test.

As in any t-test, the assumption of normality is of only moderate importance.  In the paired t-test, the assumption of normality applies to the distribution of the differences.  That is, the individual sample measurements do not have to be normally distributed, only their difference.

2.28.	The diameter of a ball bearing was measured by 12 inspectors, each using two different kinds of calipers.  The results were:

| Inspector | Caliper 1 | Caliper 2 | Difference | Difference^2 |
| --- | --- | --- | --- | --- |
| 1 | 0.265 | 0.264 | 0.001 | 0.000001 |
| 2 | 0.265 | 0.265 | 0.000 | 0 |
| 3 | 0.266 | 0.264 | 0.002 | 0.000004 |
| 4 | 0.267 | 0.266 | 0.001 | 0.000001 |
| 5 | 0.267 | 0.267 | 0.000 | 0 |
| 6 | 0.265 | 0.268 | -0.003 | 0.000009 |
| 7 | 0.267 | 0.264 | 0.003 | 0.000009 |
| 8 | 0.267 | 0.265 | 0.002 | 0.000004 |
| 9 | 0.265 | 0.265 | 0.000 | 0 |
| 10 | 0.268 | 0.267 | 0.001 | 0.000001 |
| 11 | 0.268 | 0.268 | 0.000 | 0 |
| 12 | 0.265 | 0.269 | -0.004 | 0.000016 |
|  |  |  | =0.003 | =0.000045 |

- Is there a significant difference between the means of the population of measurements represented by the two samples?  Use  = 0.05.

H0:m1=m2H1:m1¹m2  or equivalently  H0:μd=0H1:μd≠0

Minitab Output

Paired T-Test and Confidence Interval

Paired T for Caliper 1 - Caliper 2

N      Mean     StDev   SE Mean

Caliper          12  0.266250  0.001215  0.000351

Caliper          12  0.266000  0.001758  0.000508

Difference       12  0.000250  0.002006  0.000579

95% CI for mean difference: (-0.001024, 0.001524)

T-Test of mean difference = 0 (vs not = 0): T-Value = 0.43  P-Value = 0.674

- Find the P-value for the test in part (a).  P=0.674

- Construct a 95 percent confidence interval on the difference in the mean diameter measurements for the two types of calipers.

2.29.	In semiconductor manufacturing, wet chemical etching is often used to remove silicon from the backs of wafers prior to metalization.  The etch rate is an important characteristic of this process.  Two different etching solutionsare being evaluated.  Eight randomly selected wafers have been etched in each solution and the observed etch rates (in mils/min) are shown below:

|  | Solution 1 |  |  |  |  |  | Solution 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9.9 |  |  | 10.6 |  |  | 10.2 |  |  | 10.6 |  |
| 9.4 |  |  | 10.3 |  |  | 10.0 |  |  | 10.2 |  |
| 10.0 |  |  | 9.3 |  |  | 10.7 |  |  | 10.4 |  |
| 10.3 |  |  | 9.8 |  |  | 10.5 |  |  | 10.3 |  |

- Do the data indicate that the claim that both solutions have the same mean etch rate is valid?  Use a = 0.05 and assume equal variances.

No, the solutions do not have the same mean etch rate.  See the Minitab output below.

Minitab Output

Two Sample T-Test and Confidence Interval

Two-sample T for Solution 1 vs Solution 2

N      Mean     StDev   SE Mean

Solution  8     9.950     0.450      0.16

Solution  8    10.363     0.233     0.082

Difference = mu Solution 1 - mu Solution 2

Estimate for difference:  -0.413

95% CI for difference: (-0.797, -0.028)

T-Test of difference = 0 (vs not =): T-Value = -2.30  P-Value = 0.037  DF = 14

Both use Pooled StDev = 0.358

- Find a 95% confidence interval on the difference in mean etch rate.

From the Minitab output, -0.797 to –0.028.

- Use normal probability plots to investigate the adequacy of the assumptions of normality and equal variances.

Both the normality and equality of variance assumptions are valid.

2.30.	The deflection temperature under load for two different formulations of ABS plastic pipe is being studied.  Two samples of 12 observations each are prepared using each formulation, and the deflection temperatures (in F) are reported below:

|  | Formulation 1 |  |  |  | Formulation 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 206 | 193 | 192 |  | 177 | 176 | 198 |
| 188 | 207 | 210 |  | 197 | 185 | 188 |
| 205 | 185 | 194 |  | 206 | 200 | 189 |
| 187 | 189 | 178 |  | 201 | 197 | 203 |

- Construct normal probability plots for both samples.  Do these plots support assumptions of normality and equal variance for both samples?

- Do the data support the claim that the mean deflection temperature under load for formulation 1 exceeds that of formulation 2?  Use  = 0.05.

No, formulation 1 does not exceed formulation 2 per the Minitab output below.

Minitab Output

Two Sample T-Test and Confidence Interval

N      Mean     StDev   SE Mean

Form 1  12     194.5      10.2       2.9

Form 2  12    193.08      9.95       2.9

Difference = mu Form 1 - mu Form 2

Estimate for difference:  1.42

95% lower bound for difference: -5.64

T-Test of difference = 0 (vs >): T-Value = 0.34  P-Value = 0.367  DF = 22

Both use Pooled StDev = 10.1

- What is the P-value for the test in part (a)?

P = 0.367

2.31.	Refer to the data in problem 2.30.  Do the data support a claim that the mean deflection temperature under load for formulation 1 exceeds that of formulation 2 by at least 3 F?

No, formulation 1 does not exceed formulation 2 by at least 3 F.

Minitab Output

Two-Sample T-Test and CI: Form1, Form2

Two-sample T for Form 1 vs Form 2

N      Mean     StDev   SE Mean

Form 1  12     194.5      10.2       2.9

Form 2  12    193.08      9.95       2.9

Difference = mu Form 1 - mu Form 2

Estimate for difference:  1.42

95% lower bound for difference: -5.64

T-Test of difference = 3 (vs >): T-Value = -0.39  P-Value = 0.648  DF = 22

Both use Pooled StDev = 10.1

2.32.	Suppose we are testing

H0:  m1 = m2

H1:  m1 ¹ m2

where σ12  and σ22 are known.  Our sampling resources are constrained such that n1 + n2 = N.  How should we allocate the N observations between the two populations to obtain the most powerful test?

The most powerful test is attained by the n1 and n2 that maximize zo for given .

Thus, we chose n1 and n2 to   maxzo=y1−y2s12n1+s22n2, subject to n1 + n2 = N.

This is equivalent to min , subject to n1 + n2 = N.

Now ,  implies that n1 / n2 = 1 / 2.

Thus n1 and n2 are assigned proportionally to the ratio of the standard deviations.  This has

intuitive appeal, as it allocates more observations to the population with the greatest variability.

2.33.	Two popular pain medications are being compared on the basis of the speed of absorption by the body.  Specifically, tablet 1 is claimed to be absorbed twice as fast as tablet 2.  Assume that σ12 and σ22 are known.  Develop a test statistic for

H0:  2m1 = m2

H1:  2m1 ¹ m2

, assuming that the data is normally distributed.

The test statistic is:   , reject if

2.34.	Reconsider the bottle filling experiment described in Problem 2.20.  Rework this problem assuming that the two population variances are unknown but equal.

Minitab Output

Two-Sample T-Test and CI: Machine 1, Machine 2

Two-sample T for Machine 1 vs Machine 2

N      Mean     StDev   SE Mean

Machine  10   16.0150    0.0303    0.0096

Machine  10   16.0050    0.0255    0.0081

Difference = mu Machine 1 - mu Machine 2

Estimate for difference:  0.0100

95% CI for difference: (-0.0163, 0.0363)

T-Test of difference = 0 (vs not =): T-Value = 0.80  P-Value = 0.435  DF = 18

Both use Pooled StDev = 0.0280

The hypothesis test is the same:  	H0:  m1 = m2	H1:  m1 ¹ m2

The conclusions are the same as Problem 2.19, do not reject H0.  There is no difference in the machines.  The P-value for this anlysis is 0.435.

The confidence interval is (-0.0163, 0.0363).  This interval contains 0.  There is no difference in machines.

2.35.	Consider the data from problem 2.20.  If the mean fill volume of the two machines differ by as much as 0.25 ounces, what is the power of the test used in problem 2.20?  What sample size could result in a power of at least 0.9 if the actual difference in mean fill volume is 0.25 ounces?

The power is 1.0000 as shown in the analysis below.

Minitab Output

Power and Sample Size

2-Sample t Test

Testing mean 1 = mean 2 (versus not =)

Calculating power for mean 1 = mean 2 + difference

Alpha = 0.05  Sigma = 0.028

Sample

Difference    Size   Power

0.25      10  1.0000

The required sample size is 2 as shown below.

Minitab Output

Power and Sample Size

2-Sample t Test

Testing mean 1 = mean 2 (versus not =)

Calculating power for mean 1 = mean 2 + difference

Alpha = 0.05  Sigma = 0.028

Sample  Target  Actual

Difference    Size   Power   Power

0.25       2  0.9000  0.9805

2.36.	Consider the experiment described in Problem 2.22.  If the mean burning times of the two flames differ by as much as 2 minutes, find the power of the test.  What sample size would be required to detect an actual difference in mean burning time of 1 minute with a power of at least 0.90?

From the Minitab output below, the power is 0.0740.	 This answer was obtained by using the pooled estimate of σ from Problem 2.22, Sp = 9.32.  Because the difference in means is very small relative to the standard deviation, the power is very low.

Minitab Output

Power and Sample Size

2-Sample t Test

Testing mean 1 = mean 2 (versus not =)

Calculating power for mean 1 = mean 2 + difference

Alpha = 0.05  Sigma = 9.32

Sample

Difference    Size   Power

2      10  0.0740

From the Minitab output below, the required sample size is 1827.  The sample size is huge because the difference in means is very small relative to the standard deviation.

Minitab Output

Power and Sample Size

2-Sample t Test

Testing mean 1 = mean 2 (versus not =)

Calculating power for mean 1 = mean 2 + difference

Alpha = 0.05  Sigma = 9.32

Sample  Target  Actual

Difference    Size   Power   Power

1    1827  0.9000  0.9001

2.37.	Develop Equation 2.46 for a 100(1 - ) percent confidence interval for the variance of a normal distribution.

.  Thus,  .  Therefore,

,

so  is the 100(1 - )% confidence interval on 2.

2.38.	Develop Equation 2.50 for a 100(1 - ) percent confidence interval for the ratio  / σ22, where σ12 and σ22 are the variances of two normal distributions.

or

2.39.	Develop an equation for finding a 100(1 - ) percent confidence interval on the difference in the means of two normal distributions where σ12 ¹ σ22.  Apply your equation to the portland cement experiment data, and find a 95% confidence interval.

where

Using the data from Table 2.1

n1=10y1=16.764S12=0.100138          n2=10y2=17.343S22=0.0614622

where  υ=0.10013810+0.06146221020.10013810210−1+0.061462210210−1=17.024≃17

2.40.	Construct a data set for which the paired t-test statistic is very large, but for which the usual two-sample or pooled t-test statistic is small.  In general, describe how you created the data.  Does this give you any insight regarding how the paired t-test works?

| A | B | delta |
| --- | --- | --- |
| 7.1662 | 8.2416 | -1.0754 |
| 2.3590 | 2.4555 | -0.0965 |
| 19.9977 | 21.1018 | -1.1041 |
| 0.9077 | 2.3401 | -1.4324 |
| -15.9034 | -15.0013 | -0.9021 |
| -6.0722 | -5.5941 | -0.4781 |
| 9.9501 | 10.6910 | -0.7409 |
| -1.0944 | -0.1358 | -0.9586 |
| -4.6907 | -3.3446 | -1.3461 |
| -6.6929 | -5.9303 | -0.7626 |

Minitab Output

Paired T-Test and Confidence Interval

Paired T for A - B

N      Mean     StDev   SE Mean

A                10      0.59     10.06      3.18

B                10      1.48     10.11      3.20

Difference       10    -0.890     0.398     0.126

95% CI for mean difference: (-1.174, -0.605)

T-Test of mean difference = 0 (vs not = 0): T-Value = -7.07  P-Value = 0.000

Two Sample T-Test and Confidence Interval

Two-sample T for A vs B

N      Mean     StDev   SE Mean

A  10       0.6      10.1       3.2

B  10       1.5      10.1       3.2

Difference = mu A - mu B

Estimate for difference:  -0.89

95% CI for difference: (-10.37, 8.59)

T-Test of difference = 0 (vs not =): T-Value = -0.20  P-Value = 0.846  DF = 18

Both use Pooled StDev = 10.1

These two sets of data were created by making the observation for A and B moderately different within each pair (or block), but making the observations between pairs very different.  The fact that the difference between pairs is large makes the pooled estimate of the standard deviation large and the two-sample t-test statistic small.  Therefore the fairly small difference between the means of the two treatments that is present when they are applied to the same experimental unit cannot be detected.  Generally, if the blocks are very different, then this will occur.  Blocking eliminates the variability associated with the nuisance variable that they represent.